Force

Gravtional force F=G*m1*m2R2 N				Gravtional field = G*m1R2*-R     (N/kg)

Electrical force F=q1*q24*π*ε*R2*R =E*q  N                          Electrical field E= q14*π*ε*R2R  (V/m)

ε= 8.854*10-12                                                                     μ=4*π*10-7 H/m

Waves

yx,t=A*cos2*π*tT- 2*π*xλ+ ϕ0   yx,t=A*coswt- βx 							             <Opposite sign positve diection> <Same sign negative direction>

Phase velocity=u= λT = λ*f=wβ

Lossy media yx,t=A*e-αx*cos⁡(wt-βx+ϕ)

AC/DC circuit

Real battery terminal voltage ∆V= Vemf-I*r	     Kirchhoff current node law and voltage loop rule

Power I*∆V=I*Vemf= I2*R=V2R  (W)	                                      Average power= Irms2*R

sinθ=cos90-θ=cos⁡(θ-90)                         cos-θ=cos⁡(θ)

AC voltage ∆V= Vmax*sinwt	     w=2πf                  Vrms= Vmax2

Resistor in AC: i= =VmaxR*sinwt            no phase difference	     Imax =VmaxR            Irms= Imax2

Capacitor in AC: IC=w*C*∆Vmax*sinwt+π2      Current leads voltage by 90º     Imax=w*C*Vmax        XC=1w*C*j=VmaxImax

Inductor in AC: IL=Vmaxw*L*sinwt-π2        Current lags voltage by 90º			XL=w*L*j

RLC circuit maximum frequency w=1L*C

Attenuating wave: Ae-αxcoswt-βx+ϕ0→A*e-αx*ej(-βx+ϕ0)

Transmission lines

Lines affects ϕ= 2πf*lc= 2πlλ	λ=cf			Rs= π*f*μσ→surface resistance of conductor

- When lλ is very small ignore effects				c= 3*108 m/s

- When lλ>0.01, need to account for phase delay and possibly reflection

- When lλ>0..25, definitely need to account for phase delay and possibly reflection

ε0×εr= ε

L'*C'= με

G'C'=σε

Air line: ε=ε0=8.854*10-12Fm

μ=  μ0=4*π*10-7

σ=0

G'=0

Dispersion Distorts signals because different frequency components  Proportional to the length of the transmission line

Complex prepagation constant= γ= (R'+j*w*L')(G'+j*w*C')= α+ βj

Attenuation constant= α=Reγ  Np/m                                                         Phase constant= β=Imγ    rad/m

Characteristic Impedance= Z0=R'+jwL'γ=R'+jwL'G'+jwC'=V0+I0+= V0-I0-                      Phase velocity= up=wβ=f*λ

Vz=V0+e-γz+V0-e-γz

Iz=I0+e-γz+I0-e-γz

L'=βZ0w	α= R'G'=R'C'L' 	β=w*L'*C'=w*εrc

R'C'=L'G'

Lossless transmission line

R’ and G’ are negligible  α=0                    Phase velocity= up=cεr      Z0= L'C'

λ=cf*εr

β=w*εrc

Normalized load impedance= zL=ZLZ0

voltage reflection coefficient=Γ=zL-1zL+1=ZL- Z0ZL+Z0 = V0+V0-= -I0+I0-= Γ*ejθr

Wave impdeance= Zd= VdId→ratio of total voltage to total current

Wave impedance Zd=Z0*1+ Γd1- Γd, Γd= Γ*e-j*2βd                                      Input impedance= Zin=Z0*[zL+j*tan⁡(βl)1+j*zL*tan⁡(βl)]

Forward voltage= V0+=Vg*ZinZg+Zin*1ejβl+Γ*e-jβl→this part may not be used

Vector analysis

- Cylindrical <r, ϕ, z>

- Spherical <R, θ, ϕ>

- Gradient (∇)

  - ∇ =∂∂x x+∂∂y y+∂∂z z

- Divergence (∇∙A)

  - ∇∙A=divA=∂Ax∂x+∂Ay∂y+∂Az∂z

- Curl (∇ × A)

  - ∇ × A=curl A = (∂∂x x+∂∂y y+∂∂z z) ×(Axx+ Ayy+ Azz)

- Stroke’s theorem

  - ∇×B∙ds= B∙dl

- Divergence theorem

  - V∇∙EdV= SE∙ds

Gradient of cylindrical coordinate  ∂A∂r r+1r∂A∂ϕϕ+ ∂A∂z z

Gradient of Spherical coordinate   ∂A∂R R+1R*∂A∂θ θ+1Rsinθ∂A∂ϕ ϕ

Coulomb’s Law (Find electric field given charge)

E(R)=q4πε*R2 R=q4πε*R3 R  V /m→electric field at point P due to single charge

F=q'*E N→electric force on a charge placed at P

E(R)=14πε i=0Nqi*(R- Ri)R- Ri3  (V /m)

Q= v'ρvr'dV' →total charge in a volume, ρv is charge density

E(R)=v'ρv(R')(R- R')4πε R- R'3 dV'→field at point P

Infinite Plane (Disk) of charge E= ± zρv2ε	Infinite line of charge E=ρv2πεr r

Gauss’s Law (Find charge given a field)

D= εE (C/ m2)                     ε= εr*8.854*10-12	     D= Rq4*π*R2

SD∙ds'=q= v'ρv dV'→determine electric flux density D		∇∙D= ρvx,y,z→differential form

Electric Potential

V= -l'E∙dl'                                             V21= V2- V1= -P1P2E∙dl' →potential difference between P1 and P2

E∙dl'=0→for any closed path           V= -∞PE∙dl' →zero reference at infinity (free space and material media)

V (R)=14πε i=1NqiR- Ri                                  E=VL		E= -∇V

Dielectrics

- An electric dipole consists of 2-point charges of equal magnitude but opposite polarity

  - Applications: Dielectrics, molecular bonds, antennas

p=q*d→dipole moment       V (R)=p ∙ R4πε0R- Ri2=q*d*cosθ4πε0*R2

E(R)=qd4πε0R- Ri3R 2*cosθ+ θ sinθ  V/m       only when R>>d

Types of dipoles in matter

- Permanent

  - Molecule having atoms with different electronegativity

  - Polar molecule  water

- Instantaneous

  - Electrons happen to concentrate in one place

- Induced

  - A permanent dipole or applied electric field near another atom induces a dipole

In a dielectric material D= ε0E+ P→P is the electric flux density induced by applied field E

P= ε0*Xe*E           D= ε0E+ ε0*Xe*E=ε01+XeE                    1+Xe=relative permittivity= εr

Conductors & Resistors

- Conductors are materials in which some of the electrons are free electrons

  - Electrons can move relatively freely through the material

  - Copper, aluminum, and silver

  - Charge Carrier: A particle carrying charge that is free to move

Total Current I= SJ∙ds      →     J=ρv* u  Am2 →  u is velocity→ ρv=q*N, N is # of charges per unit volume

Drift velocity, u: Steady state average velocity of the electrons

Mobility µ: Accounts for the effective mass of charged particle and the average distance before stopped by colliding

ue= -μeE→drift velocity of electrons (m/s)          uh= μhE→drift velocity of holes (m/s)

J= Je+  Jh= ρe*ue+ ρh*uh=-ρe*μe +ρh*μhE                                                 J= σE (A/m2)

Semiconductor / dielectric     σ= -ρe*μe +ρh*μh= -Neqμe+Nhqμh Sm→ e(Neμe+Nhμh)

Conductor σ= -ρe*μe=Neeμe	For perfect dialectic: Ne=0, σ=0, J=0

For perfect conductor  μe= ∞, σ= ∞, E=D=0

For any conductor R=VI=-l'E∙dl'SJ∙ds=-l'E∙dl'SσE∙ds 		R=lσ1*A1+σ2*A2→resistance coaxial cable

Electric Boundary Conditions

General Boundary Conditions

E=QAε

DD: E1t= E2t    &   E2n=ε1ε2 E1n

DC: E=D=0 in perfect conductor

D1t= E1t=0      &   D1n= ε1E1n= ρs

Dielectric – dielectric	Dielectric – conductor				Conductor – Conductor

tanθ1ε1= tanθ2ε2	D1= ε1E1= ρsn			ε1*J1nσ1-ε2*J2nσ2= ρs→ J1n= J2n

Capacitor

C=QV=SεE∙ds-l'E∙dl=ε*Ad                                       RC=VI*QV=εσ		W= 0QqC dq=12*Q2C=12*C*V2=12εE2(Ad)

Q= SεE∙ds                                                 F= Q*E=12ε*A*E2 where E=Vd		W= V12εE2 dV

Magnetic Forces and torques	Force= μI1I2aπa

Magnetic force= Fm=qu × B (N)	Lorentz force= qu × B+qE		dFm=Idl × B=dQu × B

Force on any closed current loop in a uniform magnetic field=0			W= F∙dl

Magnetic torque T= m × B (N∙m) 	T=N*I*A*B*sinθ		m= nN*I*A

Biot-Savart Law

B= μ0H T	dH=NI4πR2*dl × R Am 		dB=IN μ04πR2*dl × R T

Magnetic field of a loop B=μNIa22(a2+z2)1.5 z

At z = 0 B= μ*NI2*a z

At points far away B=μNIa22z3 z

Same direction Current, attract force

Different direction current, repel force

Magnetic field of a linear conductor B=μNIa2πr4r2+a2 ϕ

R is the distance from center to point P, a is length of wire segment

For an infinity long wire B=μNI2πr ϕ

Ampere’s law (Gauss’s Law for magnetism) (net magnetic flux through a closed Gaussian surface is 0)

CH∙ dl=I= J∙ds	∇×H= J

H field for long wire:    r1≤a→H=r1*I2πa2 ϕ 		r2≥a→ H=I2πr ϕ

H field for toroidal coil:    r<a→ H=0 		r<a→ H=0  		a<r<b→  H=-NI2πr ϕ

H field inside long solenoid:   r>a→ H≈0		r<a→ H=N*IL z

H field of current sheet: z>0→H= -J2 y,  J is current density 			z<0→ H= J2 y

Magnetic vector potential & Magnetic material

B= ∇×A Wb/m2 →A is magnetic vector potential     		 A(R)= v'μ*J (R')4πR- Ri dV' (Wb/m2)

Spin Magnetic Moments: m=I*A=-e*v2πr*πr2= -e*L2m			Angular momentum: L=m*v*r

Magnetization: B= μ0H  in free space					B= μ0H+ μ0M in magnetic material

M= Xm*H→ Xm is magnetic susceptibility				M is sum of magnetic dipole moments in medium

B=μ0H+μ0 Xm*H= μ01+XmH→ 1+Xm= μr

n=NeNatoms→# of electrons per atom

Bm=μ0*M= μ0*Ne*ms

Ne=Bmμ0*ms

n=NeNatoms=Bmμ0*ms*Natoms

μ1*H1z= μ2*H2z

Inductor

L=Magnetic FluxCurrent             ϕm= sB∙ds Wb→total magntic flux         Self inductance= L=ϕmI=SB∙dsCH∙ dl  (H or Wb/A)

Self-inductance in a solenoid: ϕm=μ*N*IL*S→S is area of one loop            L=μ*N2L*S 	I=BLμN

Energy stored in solenoid: W=12*L*I2=12*B2μ	Energy density w=12μH2          Total energy in any volume W=12VμH2 dV

Self-inductance of toroid: L=μ*N22πrm*S→ rm=a+b2 				Mutual Inductance L12=N2I1S2B1∙ds

Potential energy W= 0Ii*v*dt=L0Ii*di=12LI2(J)

Faraday’s Law

- A time varying magnetic field creates transformer emf V

- A moving loop with time-varying surface area in static field B create motional emf

- A moving loop in time-varying field B is transformer emf + motional emf	V=AwB0sin⁡(wt)

Vemf= -N*dϕmdt= -NddtSB∙ds (V) 	B0=VA*w

CE∙dl = -S∂B∂t∙ds 	∇×E=-∂B∂t		V= -Addt(B0coswt+α0)

Lenz’s Law

- The current in the loop is always in a direction that opposes the change of magnetic flux that produced I

Moving Conductor in a static magnetic field

Motional EMF= V12= 21(u × B)∙dl

u × B=ux × B0z= -uB0y 	Vemf=-uB0L

Vemf= -SdBdt∙ds+ 21(u × B)∙dl

Transformers

V1V2=N1N2→turns ratio→  N2> N1 Step up traernsform;N2< N1 "Step down traernsform"    	I1I2=N2N1

Rin=V1I1=(N1N2)2*RL

Zin= (N1N2)2*ZL

Vemf= -N*dϕmdt=A*w*B0*sin⁡(wt+C0)

DC Generators: Same components as AC generator, main difference is contacts to the rotating loop are made using a split ring called commutator.

Motor: Electrical to mechanical energy        Generators works opposite

Motors are devices into which energy is transferred by electrical transmission while energy is transferred out by work

EM Motor: A current is supplied to the coil by a battery and the torque acting on the current carrying coil causes it to rotate

- Induced back emf, acts to reduce the current in the coil

- The back emf increases in magnitude as the rotational speed of coil increases

- I=Vapp-VemfR

The displacement current

∇ × H= J+∂D∂t→ ∂D∂t= Jd→displacement current density  		I=εAd*dVdt	R=dσA

CH∙ dl=SJ∙ds'+ S∂D∂t∙ds'= IC(conduction current)+ ID(Displacement current)=I

In perfect conducting wire: I1= I1c+I1d=-CV0wsin(wt)

In perfect conducting capacitor:

I2= I2c+I2d=-εAdV0wsinwt=I1

Continuity of current flow through the circuit

- The displacement current behaves like a real current

- The displacement current accounts for polarization in the medium

- The perfect wire has infinite conductivity

  - If it has finite conductivity, then D in the wire would be non-zero and I1 would consist of both conduction and displacement currents

- A magnetic field can be produced either by currents or by changing electric fields

∇∙∇×H= ∇∙J+∂∇D∂t=0	Continuity equation: ∇∙J=-∂ρ∂t					        SJ∙ds'=0 for steady currents= iIi=0

- An electric field can be produced either by charges or changing magnetic fields

EM waves

Maxwell’s equations in free space   ρV=0 &  J= 0

Maxwell’s equations in a dielectric

Maxwell’s equations in a conductor

σ is area charge density ;J is current density; ρ is charge density

Line charge

E= ρ2πεr                                                          V=ρ4πεlnl2+4r2- ll2+4r2- l, r is the distance             	   	 B=μNI2πr

Ring charge

E=λ*2πr*z4πε(x2+r2)1.5                                             B=μNIr22(r2+z2)1.5

Disk charge

E=12ε*σ*(1-zz2+r2)

Infinite plane charge

E= σ2ε                                                             B=μJ2

Point charge

E=Q4πεr2

Toroidal coil

r<a→ H=0                                              r<a→ H=0  					a<r<b→  H=-NI2πr ϕ

Long Solenoid

r>a→ H≈0                                              r<a→ H=N*IL z

Long Wire

r1≤a→H=r1*I2πa2 ϕ                                   r2≥a→ H=I2πr ϕ

Sphere

Constants

Charge of an electron e=1.602*10-19C	Electrical permittivity ε= 8.854*10-12F/m

Magnetic permittivity μ= 4π*10-7 H/m	Gravitational constant G= 6.67*10-11 N*m2/kg2

Electron mass m=9.11*10-31 kg	Proton mass m= 1.67*10-27kg

Light speed c= 3*108 m/s

Theory

Electrical Properties of particles

- SI unit of charge: coulomb <named for a French physicist, Charles-Augustin de Coulomb>

- Represents the charge on 6.241*108 electrons		Charge of an electron e=1.602*10-19C

- Charge conservation: cannot create or destroy charge, only transfer

- Superposition: vector sum of forces due to each point charge to get total force or field

- Coulomb’s experiments show <like charge repel> <force acts along the line joining the charges> <Force proportional to charges>

Magnetic field

- Individual electric charges can be isolated but magnetic poles always exist in pairs

- Magnetic fields are induced by moving charges

- Static condition: charges are stationary or moving with constant velocity

- Under static condition, electric and magnetic fields are independent. In dynamic conditions, they become coupled

Waves

- Types of waves <Circular waves> <Plane and cylindrical waves> <Spherical wave>

- A medium is said to be lossless if it does not attenuate the amplitude of the wave traveling within it or on its surface

- The phase velocity is the velocity of the wave pattern as it moves through the medium

DC & AC

- In DC, we assume <The current to be steady state> <Wires have negligible resistance>

- In AC, we assume <The current has varying magnitude and direction> <negligible resistance> <Kirchhoff’s law applies>

- Capacitor is a device that stores charge separation

- Inductor, creates strong magnetic flux for a given current and opposes changes in current flow

- RMS current, average value of current

- Phasor is a complex number in polar form, simplifies the process of solving the equation

- Impedance: accounts for the AC resistance and the phase change

Transmission lines

- Encompass all structure and media that serve to transfer energy or information between two points

- A system of conductors that transfer electromagnetic signals form one place to another

- Transverse electromagnetic: electric and magnetic fields are orthogonal to one another, and bother are orthogonal to direction

- Characteristic impedance: the ratio of forward voltage wave to forward current wave or the negative ratio of the backward traveling voltage wave term to the backward current wave term

- Lossless transmission line: R’ and G’ are negligible

- Voltage standing wave ratio, S, varies from 1 to infinity, describes the strength of the voltage amplitude standing wave pattern

- Wave impedance, ratio of the total voltage to the total current

- Dispersive: wave velocity is not constant as a function of frequency. Has distortion of the wave shape

Vector analysis

- Divergence: If the field has positive divergence then the net flux out of surface S is positive, which can be viewed as if volume contains a source of field lines

- If E has negative divergence then V may be viewed as containing a sink of field lines

- If E is uniform the same amount of flux enters V as it leaves it, the divergence is 0

- Curl: describes the rotational property, curl B is the circulation of B per unit area, with the area s of the contour C being oriented such that the circulation is maximum

Electrostatics and Magnetostatics

- Electric field/flux and electric charge are connected by Coulomb’s law

- Magnetic field/flux and electric current are connected by Biot-Savart law

- Under general case (time varying conditions): charges move around and are accelerated and there’s coupling between fluxes

- Static condition: Charges are permanently fixed in space, move at a steady rate, electric and magnetic fields de coupled

- Coulomb’s law: show the E field is constant, find electric field given charge

- Gauss’s law: Find charge given a field, determine the electric flux density (D) when charge distribution possesses symmetry

- Electric scaler potential: Amount of work or potential energy required to move a unit of charge from one point to another

  - Path independence in a conservative electric field, work is only presented when there’s work being done against the electric field, work is dependent whether it is along or going against the electric field

- Electric dipoles: consists of two-point charges of equal magnitude but opposite polarity. Applications: dielectrics, antennas

- Types of dipoles: Permanent <One atom in molecule attracts electrons more than the other, a polar molecule> Instantaneous<electrons happen to concentrate in one place, a temporary dipole> Induced <A permanent dipole or applied electric field near another atom/molecule induces a dipole, repels the electrons of the other molecule>

- Induced electric field in the material (Polarization field) is weaker than and opposite in direction to the applied electric field

- Conductors <materials in which some of the electrons are free electrons>

- Charge carriers <particle carry charge that is free to move, in a conducting medium, an electric field can exert force on these free particles, causing a net motion of the particles through the medium  current>

- Drift velocity: Steady state average velocity of the electrons.

- Mobility: Accounts for the effective mass of a charged particle and the average distance over which the applied electric field can accelerate it before it is stopped by colliding with an atom and then starts accelerating again

- Conductivity: A measure of how easily electrons can travel through the material under the influence of an externally applied electric field. For perfect dielectric, conductivity is 0. For perfect conductor, conductivity is infinity

- Electrostatic boundary conditions: at conductor boundary, E field direction is always perpendicular to conductor surface. At dielectric boundaries, electric field changes angle (it undergoes refraction)

- Capacitor: Types <Ceramic disk, film, electrolytic>

- Electrostatic potential energy: Work done in piling up charge onto plates of capacitor, energy stored in the electric field of the capacitor

- Current loops in magnetostatics: Must have closed path, otherwise charge would pile up. Total magnetic force on any closed current loop in a uniform magnetic field is zero, but there can be a force acting on a segment of the wire loop.

- Torque: acts on a closed current loop that causes the loop to rotate around its axis. It is maximum when the magnetic field and the surface normal of the loop are perpendicular

- Biot Savart law: Allows us to calculate the magnetic field due to a current carrying wire, surface current density or volume current density. Current loops create magnetic dipoles. Nearby current carry wires attract or repel due to forces.

- Ampere’s law states that the line integral of H around a closed path is equal to the current I traversing the surface bounded by the path. Like Gauss’s electrostatic law, can easily solve this involve symmetry

- Macroscopic magnetic properties: Diamagnetic, paramagnetic or ferromagnetic

- Magnetic vector potential: magnetic analogue of the electric scalar potential. Magnetization is the analogue of the electric polarization, and the magnetic permeability is a key material property

- Self-inductance: The ratio of the total magnetic flux through the cross-sectional area of the structure to the current flowing through it

- Mutual inductance: Magnetic coupling between two conducting structures. It is defined as the total flux through the second structure divided by the current in the first. And energy is stored in the magnetic field.

- Motional EMF: Only those segments of the circuit that cross magnetic field lines contribute to emf.

- EM motors: A current is supplied to the coil by a battery and the torque acting on the current carrying coil causes it to rotate

- Displacement current: does not carry free charges, it behaves like a real current, accounts for polarization in the medium

- Maxwell equations: Electric field can be produced either by charges or changing magnetic field

  - A magnetic field can be produced either by currents or changing electric field

  - Most of the E and H waves propagate in the dielectric

  - Current propagates near surface of conductor